POSITAL Rotary Encoder - How to

- Connect all rotary encoder (data and power)

- Server needs to be set in the correct IP range (take the IP range form the rotary encoder)

- Copy the folder d3driver_rotary_encoder onto your desktop

- Open a cmd window for every rotary encoder you want to use (therefor type cmd.exe)

- Type “cd desktop” (to navigate to your desktop)

- Type “cd d3driver” (to navigate to your folder)

- Type “d3driver.exe” (to run the exe)

- Now you should see the following message:

- To start the connected rotary encoder you need to assign the

  - source IP (the rotary encoder)

  - source Port (as shown on the rotary encoder)

  - destination IP (the server)

  - destination Port (you can set it as you wish, but keep it in mind)

  - driver ID (just set a number e.g. 1)

- Your command will look like e.g. this:

  - d3driver.exe 10.10.10.20 6000 10.10.10.47 6000 1

- Hit enter and keep the window open(!) and start d3

- First step in d3 is to create the object you wanna track or move (e.g. surface 1)

- Now you can create a position receiver

- In there you create a new driver

- Now you get a list of a ton of possible drivers. In our case we select the navigator driver

- In the window of the navigator driver you only have to set your port (e.g. 6000). Keep everything else as it is

- Now you create a axes (so d3 knows where and how to map the incoming data on the axis)

- In this axes window you need to engage

- Set the before selected ID (from your cmd)

- Select an object

- Type in the property (e.g. to control the surface 1 on the x axis you select surface 1 as the object and type in the property “offset.x”

  - see also: https://help.disguise.one/en/Content/Configuring/Expressions/Expressions.htm

- Last step is to engage the position receiver and have fun

Info: you can adjust/map/warp all incoming data in the axis window

Info: